Appendix D	Table S2 Recovery of phages and viruses employed as whole process controls (WPC) from different water matrices from selected studies. Viral concentration (primary and/or secondary) and elution methods, and the sampling volumes are presented. Asterisk (*) denotes onsite concentration of samples from different water matrices.

ELECTRONEGATIVE FILTRATION

| SAMPLING VOLUME (L) | MATRIX | CONCENTRATION AND ELUTION DETAILS |  |  |  | PROCESS CONTROLS |  | VIRADEL RECOVERY % | REFERENCES |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PRECONDITIONING | SECONDARY | TERTIARY | ELUANT | TYPE OF PROCESS CONTROL | WHOLE PROCESS CONTROL |  |  |
| 60L | Treated wastewater | 0.25 M AlCl3 was passed through a 0.45-μm HA filter (Sigma-Aldrich, France) | CeUF Amicon | none | NaOH | WPC | Armored-RNA Quant (ARQ) | 69.77% | (Nour et al., 2021) |
| 60L | Treated wastewater | 0.25 M AlCl3 was passed through a 0.45-μm HA filter (Sigma-Aldrich, France) | CeUF Amicon | none | NaOH | WPC | RotaV | 64.25% | (Nour et al., 2021) |
| (40 L) | River water | MgCl2 addition upto 25 mM concentration | CeUF Centricon | none | NaOH | WPC | PolioV | 8.8-10 | (Hata et al., 2015) |
| 10 to 15, mean 14 |  |  | none | none | acid rinse–alkaline elution | Whole process control, MS2 added after elution; Molecular control (CGMMV) (RNA extrcation and PCR); AdV type 5 (DNA extraction and PCR) | #MS2, CGMMV | WPC, MPC ≥10% | (Canh et al., 2021) |
| 10 to 15, mean 14 |  |  | none | none |  | Whole process control, MS2 added after elution; Molecular control (CGMMV) (RNA extrcation and PCR); AdV type 5 (DNA extraction and PCR) | #MS2, CGMMV | WPC, MPC ≥10% | (Canh et al., 2021) |
| 40L |  | none | none | none | NaOH | WPC | PV | 2.9-100 | (Hata et al., 2015) |
| 5-6 L |  |  | MgCl2 flocculation-electropositive filtration | none | Lysis buffer | Whole, molecular ( RNA extraction efficiency) | !MNV | 0.005–1.8% | (Miura et al., 2019 |
| 5-6 L |  |  |  | none |  | Whole, molecular ( RNA extraction efficiency) | !MNV | 7.4–68% | (Miura et al., 2020 |
| 5-6 L |  |  |  | none |  | Whole, molecular ( RNA extraction efficiency) | !MNV | WPC 9-54% | (Miura et al., 2021 |
| 5-6 L |  |  |  | none |  | Whole, molecular ( RNA extraction efficiency) | !MNV | 0.005–1.8% | (Miura et al., 2022 |
| 5-6L |  | after 2 conc, membrane used directly, not eluted. | Electronegative filtration | noe | H2SO4-NaOH | Whole, molecular ( RNA extraction efficiency) | !MNV | WPC 9-54% | (Miura et al., 2023 |
| 20L |  | no elution | none | none | none | WPC | !MNV | 1-100% (average 25%) for groundwater samples; 12% to 86% (GM, 39%) while method development for |  |
| 0.5-2 L |  | preconditioning by preacidification | none | none | none | WPC | na | HF183 41 - 51%, CPQ_056 38- 57%, PMMoV 19-32% | (Ahmed et al., 2019) |
| 750 mL | Groundwater | HCl (1 mol l)1) was used to adjust the water sample pH to 3.5 | none | none | (1% beef extract, 3% Tween-80 and 0.5M NaCl | WPC | Somatic coliphage | 77 ± 26% | (Helmi et al., 2011) |
| 750 mL | Groundwater | HCl (1 mol l)1) was used to adjust the water sample pH to 3.5 | none | none | (1% beef extract, 3% Tween-80 and 0.5M NaCl | WPC | F-specific coliphages | 66 ± 33% | (Helmi et al., 2011) |
| (20 L) | Milli-Q water | MgCl2 addition upto 25 mM concentration | CeUF Centricon | none | NaOH | WPC | PV | 12% | (Hata et al., 2015) |
| (1,000 L) |  | MgCl2 addition upto 25 mM concentration | CeUF Centricon | none | NaOH | WPC | PV | 13-16% | (Hata et al., 2015) |
| 150-500, average 365L |  |  | none | none | acid rinse–alkaline elution method, elution same direction as filtration | WPC, MPC Whole process control, MS2 added after elution; Molecular control (CGMMV) (RNA extrcation and PCR); AdV type 5 (DNA extraction and PCR) | #MS2, CGMMV | WPC, MPC ≥10% | (Canh et al., 2021) |
| 150-500, average 365L |  |  | none | none | acid rinse–alkaline elution method, elution same direction as filtration | WPC, MPC Whole process control, MS2 added after elution; Molecular control (CGMMV) (RNA extrcation and PCR); AdV type 5 (DNA extraction and PCR) | #MS2, CGMMV | WPC, MPC ≥10% | (Canh et al., 2021) |
| 5 L |  | none | none | none | NaOH | WPC | PV | 46% | (Hata et al., 2015) |
| 5 L |  | none | none | none | NaOH | WPC | PV | 55% | (Hata et al., 2015) |
| 750 mL |  | HCl (1 mol l)1) was used to adjust the water sample pH to 3.5 | none | none | (1% beef extract, 3% Tween-80 and 0.5M NaCl | WPC | F-specific RNA bacteriophages and somatic coliphage | Somatic : 77 ± 17%, F specific 65 ± 23% | (Helmi et al., 2011) |

ELECTROPOSITIVE FILTRATION

| SAMPLING VOLUME (L) | MATRIX | CONCENTRATION AND ELUTION |  |  |  | PROCESS CONTROLS |  | VIRADEL RECOVERY % | REFERENCES |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PRECONDITIONING | SECONDARY | TERTIARY | ELUANT | TYPE OF PROCESS CONTROL | WHOLE PROCESS CONTROL |  |  |
| 5 L | Surface water |  | none | none | Glycine beef extract (pH 11+ Tween 80, overnight) | WPC | Transmissible gastroenteritis virus (TGEV) and HAV | Ads:45.6, Elu:86.7 | (Blanco et al., 2019) |
| 710 |  |  | organic flocculation | Centrifugation (VivaspinR concentrators) | Beef extract | Whole process control, added to the water before leution. | #MEngo | <5% for WPC | (Salvador et al., 2020) |
| 20L |  |  | pH flocculation-centrifugation | Ultracentrifugation | 1% beef extract, glycine, Tween 80 , sodium polyphosphate | WPC, MPC, qPCR | AdV | WPC 18-42% , MPC and qPCR : ΔCt <2 | (Prevost et al., 2016) |
| 500 mL | Ground water | none | none | none | (1% beef extract, 3% Tween-80 and 0.5M NaCl | WPC | F-specific RNA bacteriophages and somatic coliphage | Somatic : 89 ± 8%, F specific 22 ± 14% | (Helmi et al., 2011) |
| 2713 (1088–5065L) | Drinking water |  | pH flocculation-centrifugation | Ultracentrifugation | 1% beef extract, glycine, Tween 80 , sodium polyphosphate | WPC, MPC, qPCR | AdV | WPC 18-42% , MPC and qPCR : ΔCt <2 | (Prevost et al., 2016) |
| 500 mL |  |  | none | none | none | WPC | !MNV | Filter 1 Norovirus GI undil 43.13 ± 15.53 10-fold diluted 68.61 ± 28.95 Norovirus GII Pure 15.70 ± 7.15 10-fold diluted 21.30 ± 10.78 HAV Pure) 4.18 ± 2.05 10-fold diluted 6.03 ± 1.09 MNV-1 19.75 ± 11.05 | (Hennechart-Collette et al., 2020) |
| 500 mL |  |  | none | none | none | WPC | !MNV | Filter 1 Norovirus GI undil 43.13 ± 15.53 10-fold diluted 68.61 ± 28.95 Norovirus GII Pure 15.70 ± 7.15 10-fold diluted 21.30 ± 10.78 HAV Pure) 4.18 ± 2.05 10-fold diluted 6.03 ± 1.09 MNV-1 19.75 ± 11.05 | (Hennechart-Collette et al., 2020) |
| 500 mL |  | none | none | none | Beef extract, Tween-80, NaCl | WPC | F-specific RNA bacteriophages and somatic coliphage | Somatic: 68 ± 21%, F speicifc 13 ± 9% | (Helmi et al., 2011) |
| 500 mL |  |  | none | none | none | WPC | !Noro MNV | Filter 3 Norovirus GI Undil 62.41 ± 7.86 10-fold diluted 125.89 ± 31.88 Norovirus GII Undil 37.42 ± 18.33 10-fold diluted 54.10 ± 4.44 HAV Undil 3.47 ± 0.64 10-fold diluted 4.55 ± 0.80 MNV-1 33.75 ± 14.43 | (Hennechart-Collette et al., 2020) |

Copyright of UKWIR

Copyright of UKWIR

ULTRAFILTRATION

| SAMPLING VOLUME (L) | MATRIX | CONCENTRATION AND ELUTION |  |  |  |  | PROCESS CONTROLS |  | VIRADEL RECOVERY % | REFERENCES |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PRECONDITIONING | PRIMARY | SECONDARY | TERTIARY | ELUANT | TYPE OF PROCESS CONTROL | WHOLE PROCESS CONTROL |  |  |
| 40 L | Reclaimed water | none | Dead end ultrafiltration | Celite precipitation | none | Tween 80, NaPP,AntifoamY-30 | WPC | PP7 phages | 7.81-111.99% for PP7 bacteriophage | (Prado et al., 2019) |
| 10L |  |  |  | PEG precipitation | none |  | Whole process control | PRD1, SP phages | 86-139% PRD1, 90-127% SP by qPCR | (Bailey et al., 2018) |
| 10L |  |  |  | PEG precipitation | none |  | Whole process control | PRD1, SP phages | 86-139% PRD1, 90-127% SP by qPCR | (Bailey et al., 2018) |
| 10 L | Treated wastewater | the membrane was washed with 1 L of 0.01% sodium polyphosphate as described previously (Hill et al., 2005) | TFUF | PEG precipitation | none | Beef extract | WPC | #MEngo | 10-100% | Critical Evaluation of CrAssphage as a Molecular Marker for HumanDerived Wastewater Contamination in the Aquatic Environment |
| 10L |  | the membrane was washed with 1 L of 0.01% sodium polyphosphate as described previously (Hill et al., 2005) |  | PEG precipitation | none |  | WPC | #MEngo | WPC % recovered (SD) | (Farkas et al., 2018a) |
| 10L |  | the membrane was washed with 1 L of 0.01% sodium polyphosphate as described previously (Hill et al., 2005) |  | PEG precipitation | none |  | WPC | #MEngo | 47.33 (34.12) | (Farkas et al., 2018a) |
| 10 L | River water | the membrane was washed with 1 L of 0.01% sodium polyphosphate as described previously (Hill et al., 2005) |  | PEG precipitation | none | Beef extract | WPC | #MEngo | 10-100% | Critical Evaluation of CrAssphage as a Molecular Marker for HumanDerived Wastewater Contamination in the Aquatic Environment |
| 10L |  | the membrane was washed with 1 L of 0.01% sodium polyphosphate as described previously (Hill et al., 2005) |  | PEG precipitation | none | Beef extract | WPC | #MEngo | WPC % recovered (SD) | (Farkas et al., 2018a) |
| 10L |  | the membrane was washed with 1 L of 0.01% sodium polyphosphate as described previously (Hill et al., 2005) |  | PEG precipitation | none | Beef extract | WPC | #MEngo | 59.46 (40.6) | (Farkas et al., 2018a) |
| 10 L |  | the membrane was washed with 1 L of 0.01% sodium polyphosphate as described previously (Hill et al., 2005) |  | PEG precipitation | none | Beef extract | WPC | #MEngo | 10-100% | (Farkas et al., 2018b) |
| 100 mL | DW (n=42) and PBS | Each water sample was treated with 0.5 mL/L of a 10% w/v solution of sodium thiosulfate (if chlorine may have been present) and 10% sodium polyphosphate (NaPP) prior to concentration. |  | CeUF Centricon | none | Tween® 80, NaPP, Antifoam Y-31 | WPC | #MS2 | 109% | (Mapp et al., 2016) |
| 100 L |  | Each water sample was treated with 0.5 mL/L of a 10% w/v solution of sodium thiosulfate (if chlorine may have been present) and 10% sodium polyphosphate (NaPP) prior to concentration. |  | CeUF Centricon | none |  | WPC | #MS2 | 48% | (Mapp et al., 2016) |
| 100 mL |  | Each water sample was treated with 0.5 mL/L of a 10% w/v solution of sodium thiosulfate (if chlorine may have been present) and 10% sodium polyphosphate (NaPP) prior to concentration. |  | CeUF Centricon | none |  | WPC | #MS2 | 110% | (Mapp et al., 2016) |
| 40 L |  | Each water sample was treated with 0.5 mL/L of a 10% w/v solution of sodium thiosulfate (if chlorine may have been present) and 10% sodium polyphosphate (NaPP) prior to concentration. |  | CeUF Centricon | none |  | WPC | #MS2 | 38% | (Mapp et al., 2016) |

PRECIPITATION/ FLOCCULATION

| SAMPLING VOLUME (L) | MATRIX | CONCENTRATION AND ELUTION |  |  |  |  | PROCESS CONTROLS |  | VIRADEL RECOVERY % | REFERENCES |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PRECONDITIONING | PRIMARY | SECONDARY | TERTIARY | ELUANT | TYPE OF PROCESS CONTROL | WHOLE PROCESS CONTROL |  |  |
| 60L | Treated wastewaer |  | PEG preicpitation-centrifugation | none | none | none | WPC | Adeno | 9.70% | (Nour et al., 2021) |
| 10L | Lake water | pH 3.5 | Skimmed milk flocculation- Centrifugation | none | none | none | WPC | Noro GI | 48 ± 2 | (Borgmästars et al., 2021) |
| 10L |  | pH 3.5 |  | none | none | none | WPC | #MEngo | 48 ± 2 | (Borgmästars et al., 2021) |
| 10L |  | none | Lanthanum Flocculation- Centrifugation | none | none | none | WPC | Noro GI | 17 ± 2 | (Borgmästars et al., 2021) |
| 10L |  | none |  | none | none | none | WPC | #MEngo | 17 ± 2 | (Borgmästars et al., 2021) |
| 10L | Drinking water | pH 3.5 | Skimmed milk flocculation- Centrifugation | none | none | none | WPC | Hep A | 110 ± 13 | (Borgmästars et al., 2021) |
| 10L |  | none | Lanthanum Flocculation- Centrifugation | none | none | none | WPC | Hep A | 59 ± 48 | (Borgmästars et al., 2021) |
| 450 mL | Stream water |  | Ultracentrifugation | none | none | none | WPC | na | na | (Hartard et al., 2016) |
| 500 mL | Spiked bottled water |  | OTHERS Filter 2: Durapore uncharged membrane filter (HVLP) (Hydrophillic Polyvinylidene Difluoride), pore size of 0.45 μm (Sigma Aldrich, HVLP 04700). | none | none | none | WPC | MNV | Filter 2 Norovirus GI Undil 13.43 ± 2.24 10-fold diluted 8.00 ± 7.91 Norovirus GII Undil 12.24 ± 1.44 10-fold diluted 13.33 ± 3.09 HAV Undil 2.64 ± 0.74 10-fold diluted 9.45 ± 13.69 MNV-1 15.78 ± 8.95 | (Hennechart-Collette et al., 2020) |
| 10L |  | pH 3.5 | Skimmed milk flocculation- Centrifugation | none | none | none | WPC | #MEngo | 37 ± 13 | (Borgmästars et al., 2021) |
| 10L |  | none | Lanthanum Flocculation- Centrifugation | none | none | none | WPC | #MEngo | 25 ± 10 | (Borgmästars et al., 2021) |